These are open class rules in which anything not specifically restricted or prohibited is permitted.

A. 	GENERAL

A.1 	This class is administered by the Model Yachting Association.

A.2 	Except where stated, these class rules shall be read in conjunction with World Sailing’s current Equipment Rules of Sailing (ERS). Except where used in headings, a term is printed in ‘bold’ when the definition in the ERS applies.

B. 	PRIMARY MEASUREMENT RULE

B.1 	With the exception of B.2, B.3, B.4, the boat, in racing trim, shall be capable of fitting into an open topped rectangular measurement box of internal dimensions 940mm(37") long, 229mm(9") wide and 279mm(11”) deep.

B.2 	The rig, sail(s) and hull fittings, including such parts as hatch coamings, radio gear containers and related removable hatch cover(s) may project above the plane of the top of the measurement box.

B.3 	The rudder(s) may be rotated about the normal axis (axes) of rotation and lowered along the axis (axes) of rotation with the shaft(s) within the trunking(s), for fitting within the measurement box.

B.4 	Boats measured before 31st December 1983, may continue to comply with the class rules in force at that time (rudders not included within the measurement box), but shall comply with Sections C to G of these rules.

C. 	CONDITIONS FOR RACING

C.1 	More than two channels of radio control are prohibited.

C.2 	Any other form of remote control and or automatic control is prohibited.

C.3 	During an event the following shall apply:

a) Except for boom counterbalance weights, ballast shall not be changed, moved or rotated relative to the hull.

b) Except for replacements of similar weight and position, control equipment shall not be moved, shipped or unshipped.

c) Bilge water shall not be used to trim the boat, but may be removed at any time.

D. 	HULL

D.1 	The forward 10 mm of the hull(s) shall be made of elastomeric material.

D.2 	Fittings shall be no bigger than is reasonably required for their primary purpose(s).

D.3 	The hull registration number shall be clearly and legibly marked in an easily visible location on a non-removable part of the hull(s) by any of the following means: painting on, bonding in, moulding in.

E. 	HULL APENDAGES

E.1 	Except for rudders, the following are prohibited: retractable and or movable hull appendages.

E.2 	Rudder thickness shall not exceed 13mm.

E.3 	Materials shall not be of density higher than lead 11340 kg/m3 .

F. 	RIG

F.1 	The bowsprit spar cross section and or bumpkin spar cross section shall fit within a 20 mm diameter circle. More than one bowsprit and or bumpkin is prohibited.

F.2 	Fittings shall be no bigger than is reasonably required for their primary purpose(s).

G. 	SAIL IDENTIFICATION

G.1 	The class insignia shall be the number 36, 24 - 28mm high.

G.2 	Sail numbers on headsails shall comply with the current World Sailing Racing Rules of Sailing (RRS).

G.3 	Sail numbers on mainsails shall comply with the current RRS or the rules in force at the time of manufacture, which may have required 63mm high numbers.

A.3 	GENERAL continued

A.3.1 	Class rules and their Interpretation

(a) In the event of a discrepancy within the class rule the matter shall be referred to the MYA Technical Officer.

(b) If it is uncertain how any feature of the hull or appendages shall be measured, the Official Measurer shall seek advice from the local District Senior Measurer or MYA Technical Officer.

(c) Any interpretations concerning the class rule shall be made by the MYA Tech Team.

(d) The class rules shall not be varied by a race committee.

A.4 	ADMINISTRATION

A.4.1 	Language, registration, alterations and replacements

(a) The word "shall" is mandatory and the word "may" is permissive.

(b) The builder or the owner shall apply to the class registrar for a registration number.

(c) No more than one boat shall have the same registration number.

(d) Alterations, replacements or repairs to the boat shall be made in accordance with the class rules and shall be checked by an Official Measurer where such items are required to be measured.

A.4.2 	Measurement Forms and Validity of Certificates

(a) The Measurement Form completed and signed by the Official Measurer and owner shall be scanned and sent by email to the registrar.

(b) The registrar will check the Measurement Form and produce the certificate, which he will sign to validate it.

(c) The registrar will produce a pdf from the certificate, save it as the register record and send that electronically to the owner, who will print it for use at events.

(d) Upon request the registrar may send a paper version of the certificate to an owner by post.

(e) A boat's most recent certificate shall be its only valid certificate.

A.4.3 	Change of Ownership

(a) Change of ownership invalidates the certificate, but shall not necessitate re-measurement.

(b) The new owner shall complete and sign the section of the bottom of the invalidated certificate, preferably scan and send it by email, or alternatively send it by post to the registrar.

(c) A new certificate may then be issued to the new owner, as in A.4.2 (c) or (d).

A.5 	EQUIPMENT

A.5.1 	Replacements

(a) No appendage or ballast replacement shall be used during an event, except in cases of authentic damage or loss. All replacements shall be authorised by the race committee.

A.5.2 	Prohibited

(a) Self steering devices or electronic equipment for automatic steering or rig trimming are prohibited.

H. 	FREE SAILING YACHTS

H.1 	See separate MYA Free Sailing Class Rule Supplement.